“Trendsetters in Downhole Drilling Solutions”

WZL

Deviation Control Reamer

Description

The Wenzel Downhole Deviation Control Reamer (DCR Sub) acts as a

near bit reamer/stabilizer.  The three point sealed bearing cutters

maintain hole gauge and also function as a low torque stabilizer to

control hole deviation.  The extremely short length of this sub is possible

due to the unique patented cutter retention method.

Features / Benefits

  Sealed and lubricated reamer cutters with internal bushings

provide the greatest durability in a near bit environment.

  Cutter pins are produced from hardened steel for long life.

  Cutter types; domed button or straight ridge.

  Replaceable thrust washers are installed at each end of the

cutters.

  Cutter blocks are precision machined to ensure correct gauge.

They are manufactured from alloy steel and finished hardened

for a superior combination of strength and wear resistance.

  No bolts or screws are used to retain the reamer blocks.

Deviation Control Reamer Specifications

Imperial

Downhole	Reaming

Motor Size	Diameter

(inch)	(inch)

Number

of points

Sub

Length

(feet)

Thru Bore

(inches)

Standard Bit

Box

Approximate

Weight

(lbs.)

6 1/2

6 1/2

6 1/2

8

7 7/8

8 1/2

8 3/4

12 1/4

Metric

3

3

3

3

1.8

1.9

1.9

2.0

1.5

1.5

1.5

2.0

4 1/2 API

Regular

4 1/2 API

Regular

4 1/2 API

Regular

6 5/8 API

Regular

200

200

200

550

Downhole

Motor Size

(mm)

165

165

165

203

Reaming

Diameter

(mm)

200

200

200

311

Number

of points

3

3

3

3

Sub

Length

(m)

.56

.57

.57

.62

Thru Bore

(mm)

38

38

38

51

Standard Bit

Box

4 1/2 API

Regular

4 1/2 API

Regular

4 1/2 API

Regular

6 5/8 API

Regular

Approximate

Weight

(kg)

90

90

90

250

Specifications are based on as new condition and are subject to change without notice.

Wenzel Downhole Tools Ltd.