“Trendsetters in Downhole Drilling Solutions”

WZL

Hydraulic Fishing Jar (HJ)

Description

The Wenzel Downhole Hydraulic Jar (HJ) is designed primarily for fishing

applications.  It jars in the upward direction only.  Hydraulically operated, with impact

force controlled by the operator, the HJ is ideally suited for fishing, coring, milling or

other downhole applications.

Features / Benefits

•  Delay time is controlled by a metering device, which provides uniform hydraulic

delay over a wide range of operating temperatures even after extended periods of

repeated jarring.

•  A long splined mandrel ensures the jar is not affected by torsional forces.  These

splines are sealed and lubricated to minimize friction and provide long wear life.

•  While this jar is designed to be rugged enough for drilling applications, it is

intended for fishing, coring, and milling applications.

•  A free stroke of 4” to 6” (depending on tool size) provides an impact force to the

stuck point several times higher than the overpull force applied to the jar.

•

Standard seals in the tool are effective to 275°F (135°C).  The jar can be

dressed with seals effective to 450°F (230°C) for hot hole environments.

Operation (Jarring Up)

	HJ is activated by applying upward pull from the closed position.

	The amount of upward impact force can be changed by varying the amount

of overpull applied at surface.

	When upward overpull has been applied, the jar will fire after a timed delay.

The delay is reduced as upward force is increased.

	After the jar strikes an upward blow, re-setting is quickly accomplished by

lowering the drillstring until the jar is in the closed position.

Hydraulic Fishing Jar (HJ) Specifications

Imperial

Nominal

OD

(inch)

3.12

3.38

3.75

4.25

4.75

5.00

6.25

6.50

7.75

8.00

Length

(ft)

8.2

9.0

8.5

9.5

9.8

10.2

9.7

9.3

9.2

10.6

Thru Bore

(inch)

1.00

1.50

1.25

2.00

2.25

2.25

2.25

2.25

3.00

3.00

Tensile

Yield

(lbs)

204,000

234,000

176,790

300,800

351,900

351,900

868,100

868,100

900,000

900,000

Torsional

Yield

(ft lbs)

660

7400

9100

13,800

16,100

23,300

35,000

44,000

79,600

86,900

Max Pull

During Delay

(lbs)

41,000

50,000

60,000

70,000

75,000

85,000

130,000

150,000

220,000

240,000

Free Stroke

(inches)

6.7

3.8

4.0

3.8

6.3

6.3

6.3

6.3

6.6

6.6

Total Stroke

(inches)

11

8.6

8

8.6

11

11

11

11

11

11

9.00

Metric

11.3

3.00

1,287,900	128,800

270,000

6.1

10.5

Nominal

OD

(mm)

79

86

95

108

121

127

159

165

197

203

229

Length

(m)

2.5

2.7

2.6

2.9

3.0

3.1

2.9

2.8

2.8

3.2

3.4

Thru Bore

(mm)

25.4

38.1

31.8

50.8

57.2

57.2

57.2

57.2

76.2

76.2

76.2

Tensile

Yield

(daN)

90 700

104 500

78 600

133 800

156 500

156 500

386 100

386 100

400 300

400 300

572 800

Torsional

Yield

(N·m)

8900

10 000

12 300

18 700

21 800

31 500

47 500

59 600

107 900

117 800

174 700

Max Pull

During Delay

(daN)

18 200

22 200

26 700

31 100

33 400

37 800

57 800

66 700

97 900

106 700

120 100

Free Stroke

(mm)

170

96

102

97

160

160

159

159

168

168

154

Total Stroke

(mm)

279

217

203

217

281

281

279

279

279

279

267

Specifications are based on as new condition and are subject to change without notice.

Wenzel Downhole Tools Ltd.